# SPT Kernels bitexact model for matlab

| Abstract: |  |  |
| --- | --- | --- |
| Keywords: |  |  |
| ApprovAL TABLE |  |  |
| Author | Sign-off Signature #1 Approval date | Sign-off Signature #2 Approval date |
| Mihail Catalin Mircea | Daniel Silion - 9/18/2017 |  |

REVISION HISTORY

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 9/15/2017 | Mihail Catalin Mircea | Initial version |
| 1.1 | 10/31/17 | Daniel Silion | Added support for 1024 samples x 128 chirps |
| 1.2 | 01/15/18 | Mihail Catalin Mircea | Added support for DOA and TD-MIMO |
| 1.3 | 12/17/19 | Cristian Devan | Modified the SPT kernels to work with Model-Based Design SPT Toolbox V 1.3.0 Updated kernel profiling times |
| 1.4 | 07/23/20 | Cristian Devan | Modified the SPT kernels to work with Model-Based Design SPT Toolbox V 1.4.0 |
| 1.5 | 06/11/21 | Daniel Silion | Added reference to SPT3.1 kernels |

TABLE OF CONTENTS

SPT Kernels bitexact model for matlab	1

1 Introduction	4

1.1	Scope	4

1.2	Audience Description	4

1.3	References	4

1.4	Definitions, Acronyms, and Abbreviations	4

1.5	Document Location	5

1.6	Problem Reporting Instructions	5

2.	Kernels usage and example	6

3. Profiling/execution time	7

LIST OF TABLES

Table 1 References Table	4

Table 2 Acronyms Table	4

Table 3 Kernels execution time	7

LIST OF FIGURES

No figures entries found.

# 1 Introduction

The SPT bitexact model kernels have the same functionality as RSDK SPT kernels implemented for SPT HW. Also, the output is identic with SPT_int_tester run on HW board.

Kernels can be found in src folder: rangeFFT.m, dopplerFFT.m, NonCohComb.m, FFT3D.m, peakSearch.m, DBF_DOA.m

Kernels implementation is based on SPT Toolbox [1]

## Scope

The scope of this document is to describe the functionality and usage for matlab bit-exact SPT kernels.

## Audience Description

The audience for this document is general.

## References

The documents referred across the plan can be found in the following table:

| Name | Version | location |
| --- | --- | --- |
| - Model-Based Design SPT Toolbox | v1.4.0 | NXP’s Model Based Design Community for RADAR |
| - Signal Processing Toolbox | V7.3 |  |
| - Image Processing Toolbox | V9.5 |  |

Table 1 References Table

## Definitions, Acronyms, and Abbreviations

| Term/Acronym | Definition |
| --- | --- |

Table 2 Acronyms Table

## Document Location

The document is located in rsdk\Docs\SPT\Bitexact_model\

## Problem Reporting Instructions

Problems or correction to this document should be reported to the following email address: mihailcatalin.mircea@nxp.com

# 2.	Kernels usage and example

Kernels implementation is based on SPT Matlab Toolbox [1]. For SPT Matlab Toolbox installation and usage please refer to SPT Matlab Toolbox documentation.

Kernels list:

rsdk\Tools\SPT_bitexact_model\src\SPT<n>\rangeFFT.m

rsdk\Tools\SPT_bitexact_model\src\SPT<n>\dopplerFFT.m

rsdk\Tools\SPT_bitexact_model\src\SPT<n>\NonCohComb.m

rsdk\Tools\SPT_bitexact_model\src\SPT<n>\FFT3D.m

rsdk\Tools\SPT_bitexact_model\src\SPT<n>\peakSearch.m

rsdk\Tools\SPT_bitexact_model\src\SPT<n>\DBF_DOA.m

Kernel parameters description may be found in matlab source files.

For profiling/execution time info, PROFILE_KERNEL must be set to 1.

There is an example of kernels usage:

rsdk\Tools\SPT_bitexact_model\example\SPT<n>\RSDK_offline_example.m

where n can be 2 or 3.

# 3. Profiling/execution time

The execution time of the kernels was measured in Matlab using tic/toc functions. Used case used for this: 128 chirps/frame, 512 samples/chirp, 4 physical channels..

Computer used for profiling:

Computer model: DELL Precision 7520

CPU: Intel Core i7-6820HQ@ 2.7GHz;RAM: 16GB;OS: Windows 10 64-bit Enterprise

For more detailed profile information run kernels with PROFILE_KERNEL = 1.

| Kernel - Test parameters | execution time |
| --- | --- |
| Range FFT - Chirps/frame:128 Samples/Chirp:512 | 2.02s |
| Doppler FFT - Chirps/frame:128 Samples/Chirp:512 | 2.18s |
| Non Coherent Combining - Chirps/frame:128 Samples/Chirp:512 | 1.54s |
| 3D FFT - Chirps/frame:128 Samples/Chirp:512 | 2.92s |
| Peak search - Chirps/frame:128 Samples/Chirp:512 | 0.15s |
| DBF_DOA – NoSteeringVectors: 64 NoPeaks: 128 4ch | 0.09s |

Table 3 Kernels execution time